Experiment No. 3

Aim:  To produce a Dovetail joint and Mortise joint and T-shaped joint in the carpentry shop

Materials Used: TEAK WOOOD

Size of work piece:  Dovetail joint- 125mm length, 50mm breadth, 35mm thickness.

Measuring Instruments:  1.Try square          2. steel rule

3.Marking gauge    4.Pencil

Tools required   :

1.Flat File: It is a rectangular shaped instrument . Consits  a handle attached to a blade.

2.Triangular file: it is instrument with triangular cross section

3.Square file: It is a file with square cross section.

4.Bench Vise: It is a device used to hold the object(work piece).it is made of metal ,has 2 jaws one is fixed and one is movable and a screw to adjust.

5.MALLET HAMMER: used for removing edges of a chisel.

6.Mortise Chisel: used for making strong mortise joints.

7.Wood Rasp: it is a tool consisting a round or circular cross section made of steel and a handle to hold it.

8.Jack plane:it is a tool used for cutting or removing chips from the wood.

Sequence of operation for dovetail joint:

- Take the tesk wood and mark the dimensions as shown in the drawing.

- Now cut 2 vertical work pieces of teak wood using hand saw and mortise chisel

- Take the vertical worpices and perform rough filing for both the peices.

- Cut the unnecessary material using band saw.

- Chisel the ends of the worpieces.

6.now for the dovetail part perforn wood rasp filling.

7.check weather both the pieces are fitting properly.

8 .f not perform further filing.

9.final product is achived

10.clean the workspace after use.

Result:

the dovetail joint is exactly prepared in the workshop.

Conclusion:

I learnt to make dove tail joint in the work shop sucessfully.

Sequence of operations for Mortise tail:

1.take the workpiecse and mark the dimesions as present in the drawing.

2.Check weather all marking are made and perform cutting for 2 workpiecses.

3.perform rough filing for the edges and to the required places.

4.using try square check the angle of the workpieces.

5.take the x piece and apply chalk and using vernier height guage mark required lines.

6.using dot punch make hole and remove the unwanted material using .

7.now take the y piece apply chalk and using vernier height guage mark required lines.

8.cut the y piece into desired shape.

9.remove the unwanted part using flat chisel.

10.for the stepped surface perform filing using square and triangle files.

11.now fit both of them  together.

12.if they are not fitting exactly perform filing.

13.final product is obtained.

Sequecne for T-shaped Joint:

1.take the take wood and place it inside vise to hold it .

2.clean the wood using jack plane and remove if there are any chips.

3.mark 30mm on the workpiece using height guage.

4.remove the extra part using  jack plane.

5.on the other side of the same workpiece mark 45mm and remove the unwanted part.

6.cut the work piece to our desired shape.

7.perform filing after cutting.

8.make it into 2 havles.

9.one part is corner lap joint and another is t-lap joint.

10.draw the central line for the both joints.

11.mark the lines using height guage on both the workpieces.

12.cut the unwanted material using hacksaw.

13.now using chisel and mallet hammer remove the marked material.

14.perform filing.

15.check weather the workpieces fit properly.

16.if not perform filing or cut the unwanted pieces.

17.final product is obtained.

RESULT:

T-shaped joint and Mortise tail joint are properly made.

CONCLUSION:

I learnt to make T-shaped joint and Mortise tail joint in the work shop perfectly.